# HighRisc — Ubuntu / Verilator Toolchain Command Reference

A chronological reference of the terminal commands used while setting up OSS CAD Suite, building and debugging the HighRiscSystem project under Verilator, and troubleshooting disk-space issues on Ubuntu.

## 1. OSS CAD Suite Installation & Environment Setup

Extracting the toolchain bundle and making its tools (Verilator, GTKWave, etc.) available on PATH.

| Command | Purpose |
| --- | --- |
| tar -xzf oss-cad-suite-linux-x64-20260812.tgz -C ~/Documents/Netrasemi | Extract the downloaded OSS CAD Suite archive (bundles Verilator, Yosys, GTKWave, nextpnr, etc.) into a working project directory. |
| ~/Documents/Netrasemi/oss-cad-suite/bin/verilator --version | Run Verilator via its full install path to confirm the extracted binary works, before PATH was configured. |
| which verilator | Check whether 'verilator' resolves on the shell's PATH (returned nothing until the environment script was sourced). |
| source ~/Documents/Netrasemi/oss-cad-suite/environment | Load OSS CAD Suite's bundled tools into PATH and LD_LIBRARY_PATH for the current terminal session, so bare commands like 'verilator' work directly. |
| echo 'source ~/Documents/Netrasemi/oss-cad-suite/environment' >> ~/.bashrc | Persist the environment-sourcing command so it runs automatically in every new terminal, avoiding the need to source it manually each session. |

## 2. Package Installation (apt)

System packages required for Verilator's --binary build step to compile a runnable model.

| Command | Purpose |
| --- | --- |
| sudo apt install verilator | Suggested by Ubuntu's shell when 'verilator' wasn't found. Not actually needed — the OSS CAD Suite install already provides a newer Verilator build than Ubuntu's own repository package. |
| sudo apt update | Refresh the local package index before installing new packages. |
| sudo apt install build-essential | Install make, gcc, g++ and related toolchain components required by Verilator's --binary flag, which compiles the generated C++ model into a runnable executable. |

## 3. Verilator Basics — Tutorial Sanity Checks

Working through a simple 'voter' example to confirm the toolchain functions end-to-end before touching the real project.

| Command | Purpose |
| --- | --- |
| verilator --binary voter.v voter_tb.v --top voter_tb | Attempt to build a simple tutorial testbench into a runnable binary (failed — 'voter_tb' was not the correct top-level module name declared inside the file). |
| verilator --binary voter.v voter_tb.v --top test | Corrected build using the module name actually declared inside the testbench file, found via inspection. |
| ./obj_dir/Vtest | Run the compiled simulation binary (Verilator names the output executable 'V' + the --top module name). |

## 4. GTKWave — Waveform Viewing

Producing and opening VCD waveform files.

| Command | Purpose |
| --- | --- |
| grep -n "dump" voter_tb.v | Check whether the testbench source already contains $dumpfile / $dumpvars calls needed to produce a waveform file. |
| verilator --binary --trace voter.v voter_tb.v --top test | Rebuild with waveform tracing enabled (--trace), required in addition to $dumpvars for a VCD file to actually be produced. |
| ls *.vcd | Confirm a VCD waveform file was generated after running the traced binary. |
| gtkwave voter_tb.vcd | Open the generated waveform file in GTKWave for visual inspection of signal behaviour. |

## 5. HighRiscSystem — Verilator Lint Passes

Iteratively resolving elaboration errors (missing files, module-name mismatches, unsupported constructs) before attempting a full build.

| Command | Purpose |
| --- | --- |
| ls *.sv *.v | List all SystemVerilog/Verilog source files in the working directory to confirm actual filenames before referencing them on the command line. |
| verilator --lint-only -Wall <source files> --top HighRiscTb | Run Verilator's linter-only pass (no compilation/build) across the full HighRiscSystem source set, surfacing elaboration errors and style warnings quickly before committing to a full build. |
| verilator --lint-only -Wall -Wno-fatal <source files> --top HighRiscTb | Same lint-only pass, with -Wno-fatal added so that non-fatal style/vendor-code warnings (DEFPARAM, PINCONNECTEMPTY, etc.) don't abort the run — only genuine elaboration errors do. |
| grep -n "^module" HighRiscChecker.sv HighRiscTb.sv | Check the actual module name declared inside each file, to diagnose a MODMISSING instantiation error caused by a filename/module-name mismatch. |
| grep -rn "^module RiscChecker" *.sv | Search every SystemVerilog file in the directory for wherever a specific module name is actually declared, to resolve an instantiation that Verilator couldn't find. |

## 6. HighRiscSystem — Full Build & Run

Compiling the complete processor, memory, VGA and SVA checker sources into a single simulation binary.

| Command | Purpose |
| --- | --- |
| verilator --binary --trace --assert -Wno-fatal <source files> --top HighRiscTb | Full build of the HighRiscSystem project: compiles the processor, memory, VGA controller, SVA checker and testbench into one runnable binary, with waveform tracing (--trace) and SystemVerilog Assertion evaluation (--assert) both enabled. |
| ./obj_dir/VHighRiscTb | Run the compiled full-system simulation binary directly. |
| gtkwave HighRiscTb.vcd | Open the resulting system-level waveform in GTKWave to inspect internal signals such as PC, Instruction, and bus activity. |

## 7. Safe / Diagnostic Simulation Execution

Running simulations with bounded runtime and redirected output, to avoid runaway processes and to capture results for inspection.

| Command | Purpose |
| --- | --- |
| timeout 30 ./obj_dir/VHighRiscTb > sim.log 2>&1 | Run the simulation with a hard 30-second wall-clock cutoff and redirect all console output to a log file, preventing a stalled or looping simulation from hanging the terminal indefinitely. |
| ls -la sim.log | Check the size of the captured log file to judge whether the run produced meaningful output or stalled immediately. |
| wc -l sim.log | Count the number of lines written to the log, to gauge output volume (e.g. detecting runaway $monitor spam). |
| grep -i "final value\¦assert\¦fail\¦error" sim.log | Search the captured log for key result and failure indicators without having to scroll through the entire file. |
| timeout 10 ./obj_dir/VHighRiscTb > sim_short.log 2>&1 | Shortened, safely-bounded diagnostic run used specifically for tracing the Program Counter / instruction-fetch signals over a small number of cycles. |
| echo "exit code: $?" | Print the exit status of the previous command, confirming whether the simulation terminated via $finish or was killed by the timeout. |
| head -30 sim_short.log | Preview the first 30 lines of a diagnostic log to quickly check early simulation behaviour (e.g. PC and Instruction values at time = 0). |

## 8. Disk Space & Syslog Troubleshooting

Diagnosing and clearing a syslog partition that had filled to over 40GB from simulator output being captured by the system journal.

| Command | Purpose |
| --- | --- |
| df -h / | Check root filesystem disk usage as a percentage, to confirm whether a full disk was the cause of commands failing. |
| sudo du -sh /var/log/* | List the size of each file under /var/log to identify which log file was consuming the most disk space. |
| sudo truncate -s 0 /var/log/syslog | Empty the syslog file in place without deleting it — safer than 'rm', since a process holding the file open would otherwise keep the disk space allocated regardless of deletion. |
| sudo rm -f /var/log/syslog.*.gz /var/log/syslog.* | Remove old rotated and compressed syslog backup files that were also contributing to disk usage. |
| journalctl --disk-usage | Check how much disk space the systemd journal (journald) is currently consuming. |
| sudo journalctl --vacuum-size=200M | Cap the systemd journal to a fixed maximum size, discarding older entries to reclaim disk space. |
| ls -la /var/crash | Check for accumulated crash reports, which can be written automatically by Ubuntu's crash-reporting service after a process is killed or segfaults. |
| du -sh /var/crash | Check the total size of the crash-report directory. |
| sudo rm -rf /var/crash/* | Clear all accumulated crash reports to reclaim disk space. |
| systemctl --failed | List any systemd services currently in a failed state — a common cause of tight, repeating log entries. |
| sudo journalctl -u <service-name> --since "2 minutes ago" | Inspect the most recent log entries for a specific systemd service to see the actual repeating error message. |
| sudo dmesg -T ¦ tail -50 | Check the most recent kernel-level messages for hardware or disk I/O errors, particularly after heavy write activity from a large log file. |
| free -h | Check overall system memory usage as part of general system-health triage. |

## 9. Process Management

Identifying and terminating orphaned simulation processes that were continuing to run — and log — in the background.

| Command | Purpose |
| --- | --- |
| sudo tail -f /var/log/syslog | Watch syslog in real time to identify exactly which process was actively writing to it as it happened. |
| pkill -9 -f VHighRiscTb | Forcibly terminate every running instance of the compiled simulation binary, identified by matching its process name. |
| pgrep -af VHighRiscTb | List any remaining running instances of the simulation binary, used to confirm that pkill fully cleaned them up (expected to return nothing when clean). |